These samples are being collected to determine the lead and copper levels in your drinking water. This sampling effort is required by the U.S. Environmental Protection Agency and the state and can only be accomplished with your cooperation. (Public Drinking Water Regulations [R46-13-DWQ §6.86] for exact requirements)

It is important that the samples are not contaminated and come from water that has been in the pipes for at least six hours.  When you receive sample bottles, keep them closed until collection and avoid excess handling to guard against contamination.   Do not use the water anywhere in the building for 6-8 hours (preferably overnight) before collecting a sample from an approved tap.  Collect only one sample per building during any 6 – 8 hour span.  (If your system supplies water to multiple buildings with approved sampling sites, you may sample from more than one building at a time.)

- Choose a cold water tap that you have been using for consumption on a regular basis. If possible, do not take the sample from a tap with a water softener attached.

- Be sure the tap to be sampled is in regular use during the monitoring period. DO NOT collect a sample from an unused tap.

- Wait at least six hours.  Do not use this tap, or any tap near it, while you are waiting. (Its often convenient to wait overnight.  If the water is used anywhere in the building, you must start the holding time of 6-8 hours again before collecting a sample.)

- Place the opened sample bottle below the faucet and open the cold water tap as you would to fill a glass of water. If the tap has an aerator, do not remove it prior to sampling.

- Fill the sample bottle to the bottom of the neck of the bottle (line marked “1000-mL”) and turn off the water.

- Tightly cap the sample bottle and place in the sample kit provided.

- Review the sample submission form to ensure that all information is correct.

- Refrigerate all filled sample bottles until you deliver them to the laboratory.

- Note if any plumbing repairs or replacements have been done since you last sampling event on the label  provided.  Also note if your sample was collected from a tap with a water softener.

If you have any questions about these instructions please contact the Department of Health at 401-222-6867.

Plumbing repairs done since the last monitoring:

__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________.

Please note the following information for each sample

Tap Location                      Water was last Used                              Sample was collected

(Date/Time)                                           (Date/Time)

1) ___________                    _____________________                   _________________________

2) ___________                    _____________________                   _________________________

3) ___________                    _____________________                   _________________________

4) ___________                    _____________________                   _________________________

5) ___________                    _____________________                   _________________________

6) ___________                    _____________________                   _________________________

7) ___________                    _____________________                   _________________________

8) ___________                    _____________________                   _________________________

9) ___________                    _____________________                   _________________________

10)__________                    _____________________                   _________________________

I have taken samples in accordance with these instructions and I have filled out the above time chart accurately.

System Name: ________________________________________PWSID:___________________

Your Name ___________________________________

__________________________________________ Date____________________________

Signature